REFRSH.EXE is a utility for setting the

Refresh Rate for 4 graphics resolutions:

1) 1280 Horizontal by 1024 Vertical

2) 1024 Horizontal by 768  Vertical

3) 800  Horizontal by 600  Vertical

4) 640  Horizontal by 480  Vertical

This program can be run in Command Line mode or in Interactive Mode.

The Command Line mode accepts 2 parameters:

Horizontal Resolution & Frequency.

Example:

REFRSH 800 56

The Horizontal Resolutions are the same as in 1 thru 4 above.

The Frequencies are:

84 (for Interlaced)

56 (non Interlaced)

60 (non Interlaced)

72 (non Interlaced)

75 (non Interlaced)

A help Screen with the parameters is displayed

by giving only 1 Command Line Parameter.

Example:

REFRSH ?

The Interactive Mode has NO Command Line parameters.

The program reads the current Refresh settings and displays them

in a table.  Invalid combinations are represented in the table by an X.

The current setting for each column is represented by a check mark: �.

The user can move the blinking cursor by either using the cursor keys,

or (if a mouse driver is loaded) by using the mouse.  A new Frequency

can be selected by moving the cursor to a different row and either

pressing the SPACE BAR key or clicking mouse button 1.  The old Frequency

for that column will be erased and the new one will be selected.

This Frequency can also be tested at that resolution at all the various

color depths available for the amount of video memory installed:

16, 256, 32k, 64k & 16.8M colors.  To test, either click on the Red T

in the upper left of the screen or press the T key.  (The TAB key will test

1600x1200 Interlaced). If the monitor cannot display this Frequency,

either press the ESC key or click mouse button 2 and the table

will be re-displayed for another selection.

When all the Frequencies have been selected either press the ESC key or

click mouse button 2.  The program will write a file called WDREFRSH.BAT

to the current directory.  This file contains the Command Line options of

REFRSH to select the Frequencies that were in effect when you left the program.