# Table of Content

µTable of Content	I

Chapter 1.	1

1.  SiS 6202  Overview	1

1.1  Introduction	1

1.2  Features	1

1.3  Block Diagram	3

1.3.1  SiS 6202 Demo Board Block Diagram	3

1.3.2  SiS 6202 Demo Board "02" Outline	4

1.3.2.1 Hardware Jumper Settings	4

1.3.2.2 Components Description	4

1.3.3  SiS 6202 Demo Board "03" Outline	5

1.3.3.1 Hardware Jumper Settings	5

1.3.3.2 Components Description	5

Chapter 2.	6

1. VESA Video Modes	6

Chapter 3.	7

1. Install Program: INSTDRV.EXE	7

2. SVGAUTL.EXE	9

2.1 General Description	9

2.2 Unpack & Copy	9

2.3 How to Use?	9

2.4 What the Parameters Stand for?	10

3. Video Modes	12

3.1 Standard VGA Modes	12

3.2 Enhanced Video Modes	14

Chapter 4.	17

1. Windows 3.1	18

1.1 Driver Files	18

1.2 Installation	19

1.2.1 Unpack & Copy	19

1.2.2  Graphics Setup	19

1.2.3  Power Saving Setup in Windows	20

1.2.4  Zoom_Key Setup	20

1.2.5 Notes	20

2. OS/2 V3.0  (Warp)	22

2.0	Driver & OS/2 Version Notes	22

2.1	Driver Files	22

2.2 Installation	22

3. Autodesk ADI 4.2 -Protected Mode	24

3.0 General Description	24

3.0.1 Driver Files	24

3.0.2 Unpack & Copy	24

3.1 AutoCAD R11 Setup	26

3.2 AutoCAD R12 Setup	27

3.3 AutoShade R2.0 Setup	28

3.4 3D Studio Version 3.0 Setup	29

4. WordPerfect V5.1	31

4.1 Driver File	31

4.2 Installation	31

4.2.1 Unpack & Copy	31

4.2.2 WordPerfect Setup	32

5. GEM 3.0 / Ventura 2.0	33

5.1 Driver Files	33

5.2 Installation	33

5.2.1 Unpack & Copy	33

5.2.2 Ventura Setup --- Create New VP.BAT	34

6. PCAD 6.06	35

6.1 Driver Files	35

6.2 Installation	35

6.2.1 Unpack & Copy	35

6.2.2 PCAD Setup --- Create New PCADDRV.SYS	36

7. VersaCAD/386 2.1	37

7.1 Driver Files	37

7.2 Installation	37

7.2.1 Unpack & Copy	37

7.2.2 VersaCAD/386 Setup	38

8. OrCAD Rel. 4	39

8.1 Driver Files	39

8.2 Installation	39

8.2.1 Unpack & Copy	39

8.2.2 OrCAD Display Setup	40

8.2.3 OrCAD SDT.CFG Setup	41

9. Lotus 2.x	42

9.1 Driver Files	42

9.2 Installation	42

9.2.1 Unpack & Copy	42

9.2.2 Lotus 1-2-3 Setup	43

10. Windows NT 3.1 & 3.5	44

10.1 Driver Files	44

10.2 Installation	44

10.2.1 Windows NT Setup	44

11.  Windows 95	45

11.0  Version Note	45

11.1  Driver Files	45

11.2  Installation and Using "SiS Settings"	45

12.  Full Pack Double Bytes OS/2 Warp	47

12.0  Driver & OS/2 Version Notes	47

12.1	Driver Files	47

12.2 Installation	47

COPYRIGHT NOTICE	49

# Chapter 1.

SiS 6202  Demo Board

# 1.  SiS 6202  Overview

## 1.1  Introduction

SiS has created a completely new design for the SiS 6202, which based on a new generation of advanced architecture and local bus standard, added with the characteristics of high performance, high integration, and low cost solution to form a PCI True-Color Graphics Accelerator.

## 1.2  Features

PCI Bus Interface

·	Support 32-bit PCI Local Bus Standard Revision 2.0

·	Support PCI Bus-Master Architecture

·	Support PCI Burst Write

·	Support PCI Multi-Funciton Device

·	Follow the One-Load-Per-Slot PCI Specification

Performance

·	Support PCI Bus-Master Architecture to Achieve Extra-High Performance

·	Built-in an Enhanced 32-bit BITBLT Graphic Engine with following functions:

·	256 Raster Operation Functions

·	Rectangle Fill

·	Color/Font Expansion

·	Enhanced Color/Font expansion

·	Line-Drawing with Styled Pattern

·	Built-in 8x8 Pattern Registers

·	Built-in 8x8 Mask Registers

·	24 Doublewords Command Queue

·	Built-in 64x64x2 Bit-Mapped Hardware Cursor

·	Built-in 4 Stages CPU Write-Buffer and 128 Bits Read-ahead Cache to Minimize CPU Wait-state

·	Built-in 32x24 CRT FIFOs to Support Super High Resolution Graphic Modes and Reduce CPU Wait-State

·	Memory-Mapped I/O to Reduce I/O Trapping Overhead Under Protected Mode

·	Support Linear Addressing Mode up to 4MByte to Speed Up Graphic Performance

Integration

·	Built-in Programmable 24-bit True-Color RAMDAC with Reference-Voltage Generator

·	Built-in Monitor-Sense Circuit

·	Built-in Feature Connector Logic

·	Built-in Dual-Clock Generator

·	External Dual-Clock Generator Option Available

Display Memory Interface

·	32-bit Interleaved Video Memory Data Bus Architecture up to 4 banks with 240 MByte/sec Peak Memory Bandwidth

·	Support 256Kx4, 256Kx8, and 256Kx16 DRAM Types

·	Support 1MB, 2MB and 4MB DRAM Configuration

·	Support Auto Memory Size Detecting and Auto Bank Switching

·	Support 2-CAS/1-WE and 2-WE/1-CAS 256Kx16 DRAM Types

Resolution, Color & Frame Rate

·	Support Super High Resolution Graphic Modes

- 640x480		256/32K/64K/16M colors NI

- 800x600		16/256/32K/64K/16M colors NI

- 1024x768	16/256/32K/64K colors NI, 16M colors interlace only

- 1280x1024	16/256 colors NI, 32K/64K colors interlace only

·	Support Virtual Screen Up to 2048x2048

·	Support 80/132 Columns Text Mode in 25, 30, 44 or 60 Rows and other modes

·	Support 75Hz Vertical Refresh Rate

Power Management

·	Support VESA DPMS Compliant VGA Monitor for Power Management

·	Built-in 30 min. Standby and Suspend Timers with Keyboard, Hardware Cursor and/or Video Memory Read/Write as Activation Source

·	Support Direct I/O Command to Force Graphics Controller into Standby/Suspend/Off State

·	Power Down internal SRAM in Direct Color Mode

Multimedia Application

·	Support DDC1 and DDC2B Specifications

·	Follow the Plug & Play Specification for Display Controller

·	Support RAMDAC Snoop for Multimedia Applications

Misc.

·	Only 4 ICs (including DRAMs) Required to Implement a PCI True-Color Graphics Adapter Without Any TTLs

·	Support 64K ROM Decoding

·	Implemented by Sub-Micron CMOS Technology in 160 Pins PQFP Package

## 1.3  Block Diagram

### 1.3.1  SiS 6202 Demo Board Block Diagram

µ §

### 1.3.2  SiS 6202 Demo Board "02" Outline

µ §

#### 1.3.2.1 Hardware Jumper Settings

J1: 	15-Pin VGA Monitor Connector

JP1: 	26-Pin Feature Connector

JP2: 	PCI INTA Jumper Setting (not used now)

#### 1.3.2.2 Components Description

U1: 	ROM BIOS

U2: 	SiS 6202, VGA Controller

U3 & U5: 	256Kx16 DRAM

U4 & U6: 	256Kx16 DRAM

Q1: 	2N3904

Y1: 	14.318 MHz Crystal  (Reference Clock)

### 1.3.3  SiS 6202 Demo Board "03" Outline

#### 1.3.3.1 Hardware Jumper Settings

J1: 	15-Pin VGA Monitor Connector

JP1: 	26-Pin Feature Connector

JP2: 	PCI INTA Jumper Setting (not used now)

#### 1.3.3.2 Components Description

U1: 	ROM BIOS

U2: 	SiS 6202, VGA Controller

U3 & U4: 	256Kx16 DRAM

U5 ~ U12: 	256Kx4  DRAM

Q1: 	2N3904

Y1: 	14.318 MHz Crystal  (Reference Clock)

# Chapter 2.

VESA Extension Compatible BIOS

# 1. VESA Video Modes

1. 	The VESA Extension (Version 1.2) Compatible BIOS provided by SiS supports the following VESA Video Modes:

| 15-bit mode # | Type | Resolution | Colors | Alpha Fomat | Char. Size | min. DRAM |
| --- | --- | --- | --- | --- | --- | --- |
| 100h | APA | 640x400 | 256 | 80x25 | 8x16 | 1MB |
| 101h | APA | 640x480 | 256 | 80x30 | 8x16 | 1MB |
| 102h | APA | 800x600 | 16 | 100x37 | 8x16 | 1MB |
| 103h | APA | 800x600 | 256 | 100x37 | 8x16 | 1MB |
| 104h | APA | 1024x768 | 16 | 128x48 | 8x16 | 1MB |
| 105h | APA | 1024x768 | 256 | 128x48 | 8x16 | 1MB |
| 106h | APA | 1280x1024 | 16 | 160x64 | 8x16 | 1MB |
| 107h | APA | 1280x1024 | 256 | 160x64 | 8x16 | 2MB |
| 108h | A/N | 640x480 | 16 | 80x60 | 8x8 | 1MB |
| 109h | A/N | 1056x350 | 16 | 132x25 | 8x14 | 1MB |
| 10Dh | APA | 320x200 | 32K | 40x25 | 8x8 | 1MB |
| 10Eh | APA | 320x200 | 64K | 40x25 | 8x8 | 1MB |
| 10Fh | APA | 320x200 | 16M | 40x25 | 8x8 | 1MB |
| 110h | APA | 640x480 | 32K | 80x30 | 8x16 | 1MB |
| 111h | APA | 640x480 | 64K | 80x30 | 8x16 | 1MB |
| 112h | APA | 640x480 | 16M | 80x30 | 8x16 | 1MB |
| 113h | APA | 800x600 | 32K | 100x37 | 8x16 | 1MB |
| 114h | APA | 800x600 | 64K | 100x37 | 8x16 | 1MB |
| 115h | APA | 800x600 | 16M | 100x37 | 8x16 | 2MB |
| 116h | APA | 1024x768 | 32K | 128x48 | 8x16 | 2MB |
| 117h | APA | 1024x768 | 64K | 128x48 | 8x16 | 2MB |
| 118h | APA | 1024x768 | 16M | 128x48 | 8x16 | 4MB |
| 119h | APA | 1280x1024 | 32K | 160x64 | 8x16 | 4MB |
| 11Ah | APA | 1280x1024 | 64K | 160x64 | 8x16 | 4MB |

Note:

1. APA: All Point Addressable (Graphic) mode

2. A/N: AlphaNumeric (Text) mode

# Chapter 3.

DOS Utility

# 1. Install Program: INSTDRV.EXE

1. 	Most of the SiS drivers are in packed file form.

You must unpack them before beginning installation.

2.	To help you unpack and setup SiS 6202 drivers,

SiS provides an unpack and copy program INSTDRV.EXE.

3.	INSTDRV.EXE can unpack and copy drivers to where you want them to be.

For some software packages, INSTDRV.EXE can even complete setup for you.

4. 	To use INSTDRV.EXE,

please follow the procedures below:

(1) 	Insert SiS SVGA Drivers-Setup diskette in drive A or drive B.

(2) 	Change the current disk drive to the one where you insert the diskette.

(3) 	Run INSTDRV.EXE program.

For example:

A:\>instdrv 	<Enter>

(4) 	From the main menu,

select the desired application software.

(5) 	Follow the instructions appear on the screen successively

to complete the process.

(6) 	After complete the process,

exit INSTDRV.EXE program.

(7) 	You may start your software program either

(a) using new driver just installed, or to

(b) continue the installation in order to really install the driver.

5. 	You should realize

(1) 	For some software packages (e.g. ADI packages, PCAD),

their driver files can be put to any directory you prefer.

(2) 	For some software packages (e.g. WordPerfect, GEM/Ventura,

VersaCAD/386, Lotus 1-2-3),

the driver files must be put to where main program resides.

(3) 	For some software packages (e.g. OrCAD Rel. 4),

they have their default directory for the drivers to reside.

Therefore, you should be very careful of the "drive:\directory" you assign for these drivers. Otherwise, the installation will fail.

# 2. SVGAUTL.EXE

## 2.1 General Description

SVGAUTL.EXE is one of the utilities of SiS 6202 display cards. It supports three functions:

(1) Video Mode Setting

(2) Frame Rate Setting

(3) Power Saving Setting

Since SiS 6202 supports many enhanced Text Mode and Graphic Mode,

you can use SVGAUTL.EXE to select the desired video mode.

For 720x400, 640x480, 800x600, 1024x768, and 1280x1024 resolutions,

SiS 6202 supports multiple frame rates.

If your monitor could synchronize with these frame rates,

you can use SVGAUTL.EXE to get advantage.

SiS 6202 also supports VESA DPMS Power Saving Modes.

SVGAUTL.EXE can help you to set the capability.

## 2.2 Unpack & Copy

To unpack and copy "SVGAUTL.EXE" and related files to where they should resides, please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "J. Utilities" to unpack and copy related files.

(To select, type "H".)

4. 	In "Unpack & Copy Utilities" screen,

keyin the "drive:\directory" where you would like to use.

Program would unpack & copy all related files to where you assign.

5. 	After "unpack and copy" completed,

exit INSTDRV.EXE program.

## 2.3 How to Use?

To use SVGAUTL.EXE,

1. 	Type SVGAUTL in the directory where it resides.

For example,

C:\> SVGAUTL 	<Enter>   (suppose SVGAUTL.EXE resides in C:\)

2. 	The Main Menu would appear and direct you to configure SiS 6202.

3. 	When you complete configuration, you may save your preferences to "AUTOEXEC.BAT file and use it as your power-on (or hardware reset) default environment.

## 2.4 What the Parameters Stand for?

The meaning of parameters used by SVGAUTL.EXE are explained below:

Syntax:

>SVGAUTL [/D:mode_no] [/F1:n1] [/F2:n2] [/F3:n3] [/F4:n4] [/PA:ta] [/PB:tb]

where

/D: 	Set the Video Mode to be mode_no which is a hex number.

For example: Set 1024x768 256 color graphic mode.

>SVGAUTL /D:38 <Enter>

/F1: 	For 640x480, set frame rate to be n1 Hz.

Three availabe frame rates are 60, 72, and 75 Hz.

For example : Set 640x480 graphic mode with 60Hz frame rate

>SVGAUTL /F1:60 <Enter>

/F2: 	For 800x600, set frame rate to be n2 Hz.

Four availabe frame rates are 56, 60, 72, and 75 Hz.

For example : Set 800x600 graphic mode with 72Hz frame rate.

>SVGAUTL /F2:72 <Enter>

/F3: 	For 1024x768, set frame rate to be n3 Hz.

Four availabe frame rates are 87 (Interlace), 60, 70, and 75 Hz.

For example : Set 1024x768 graphic mode with 60Hz frame rate.

>SVGAUTL /F3:60 <Enter>

/F4: 	For 1280x1024, set frame rate to be n4 Hz.

Two availabe frame rates are 87 (Interlace) and 60 Hz.

For example : Set 1280x1024 graphic mode with 60Hz frame rate.

>SVGAUTL /F4:60 <Enter>

/PA: 	Set Standby Timer to be ta minutes. (0 < ta < 15 min.)

For example : Set Standby Timer be 5 minutes.

>SVGAUTL /PA:5 <Enter>

/PB: 	Set Suspend Timer to be tb minutes. (0 < tb < 15 min.)

For example : Set Suspend Timer be 5 minutes.

>SVGAUTL /PB:5 <Enter>

Note:

1. 	Suspend Time would be "ta + tb". (i.e. standby time + suspend time)

2. 	The timers will not be very accurate and would just be around the time.

# 3. Video Modes

## 3.1 Standard VGA Modes

| MODE | TYPE | DISPLAY SIZE | COLORS SHADES | ALPHA FORMAT | BUFFER START | BOX SIZE | MAX. PAGES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | A/N | 320x200 | 16 | 40x25 | B800 | 8x8 | 8 |
| 0* | A/N | 320x350 | 16 | 40x25 | B800 | 8x14 | 8 |
| 0+ | A/N | 360x400 | 16 | 40x25 | B800 | 9x16 | 8 |
| 1 | A/N | 320x200 | 16 | 40x25 | B800 | 8x8 | 8 |
| 1* | A/N | 320x350 | 16 | 40x25 | B800 | 8x14 | 8 |
| 1+ | A/N | 360x400 | 16 | 40x25 | B800 | 9x16 | 8 |
| 2 | A/N | 640x200 | 16 | 80x25 | B800 | 8x8 | 8 |
| 2* | A/N | 640x350 | 16 | 80x25 | B800 | 8x14 | 8 |
| 2+ | A/N | 720x400 | 16 | 80x25 | B800 | 9x16 | 8 |
| 3 | A/N | 640x200 | 16 | 80x25 | B800 | 8x8 | 8 |
| 3* | A/N | 640x350 | 16 | 80x25 | B800 | 8x14 | 8 |
| 3+ | A/N | 720x400 | 16 | 80x25 | B800 | 9x16 | 8 |
| 4 | APA | 320x200 | 4 | 40x25 | B800 | 8x8 | 1 |
| 5 | APA | 320x200 | 4 | 40x25 | B800 | 8x8 | 1 |
| 6 | APA | 640x200 | 2 | 80x25 | B800 | 8x8 | 1 |
| 7 | A/N | 720x350 | 4 | 80x25 | B000 | 9x14 | 8 |
| 7+ | A/N | 720x400 | 4 | 80x25 | B000 | 9x16 | 8 |
| 0D | APA | 320x200 | 16 | 40x25 | A000 | 8x8 | 8 |
| 0E | APA | 640x200 | 16 | 80x25 | A000 | 8x8 | 4 |
| 0F | APA | 640x350 | 2 | 80x25 | B000 | 8x14 | 2 |
| 10 | APA | 640x350 | 16 | 80x25 | A000 | 8x14 | 2 |
| 11 | APA | 640x480 | 2 | 80x30 | A000 | 8x16 | 1 |
| 12 | APA | 640x480 | 16 | 80x30 | A000 | 8x16 | 1 |
| 13 | APA | 320x200 | 256 | 40x25 | A000 | 8x8 | 1 |

Note:	1. A/N: Alpha/Numeric

2. APA: All Point Addressable (Graphic)

| MODE | DISPLAY SIZE | COLORS SHADES | FRAME RATE. | H-SYNC. | VIDEO FREQ. |
| --- | --- | --- | --- | --- | --- |
| 0 | 320x200 | 16 | 70 | 31.5 K | 25.1 M |
| 0* | 320x350 | 16 | 70 | 31.5 K | 25.1 M |
| 0+ | 360x400 | 16 | 70 | 31.5 K | 28.3 M |
| 1 | 320x200 | 16 | 70 | 31.5 K | 25.1 M |
| 1* | 320x350 | 16 | 70 | 31.5 K | 25.1 M |
| 1+ | 360x400 | 16 | 70 | 31.5 K | 28.3 M |
| 2 | 640x200 | 16 | 70 | 31.5 K | 25.1 M |
| 2* | 640x350 | 16 | 70 | 31.5 K | 25.1 M |
| 2+ | 720x400 | 16 | 70 | 31.5 K | 28.3 M |
| 3 | 640x200 | 16 | 70 | 31.5 K | 25.1 M |
| 3* | 640x350 | 16 | 70 | 31.5 K | 25.1 M |
| 3+ | 720x400 | 16 | 70 | 31.5 K | 28.3 M |
| 4 | 320x200 | 4 | 70 | 31.5 K | 25.1 M |
| 5 | 320x200 | 4 | 70 | 31.5 K | 25.1 M |
| 6 | 640x200 | 2 | 70 | 31.5 K | 25.1 M |
| 7* | 720x350 | 4 | 70 | 31.5 K | 28.3 M |
| 7+ | 720x400 | 4 | 70 | 31.5 K | 28.3 M |
| 0D | 320x200 | 16 | 70 | 31.5 K | 25.1 M |
| 0E | 640x200 | 16 | 70 | 31.5 K | 25.1 M |
| 0F | 640x350 | 2 | 70 | 31.5 K | 25.1 M |
| 10 | 640x350 | 16 | 70 | 31.5 K | 25.1 M |
| 11 | 640x480 | 2 | 60 | 31.5 K | 25.1 M |
| 12 | 640x480 | 16 | 60 | 31.5 K | 25.1 M |
| 13 | 320x200 | 256 | 70 | 31.5 K | 25.1 M |

Note : 	i - interlaced mode

n - noninterlaced mode

## 3.2 Enhanced Video Modes

| MODE | TYPE | DISPLAY SIZE | COLORS SHADES | ALPHA FORMAT | BUFFER START | BOX SIZE | MAX. PAGES |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 22 | A/N | 1056x352 | 16 | 132x44 | B800 | 8x8 | 2 |
| 23 | A/N | 1056x350 | 16 | 132x25 | B800 | 8x14 | 4 |
| 24 | A/N | 1056x364 | 16 | 132x28 | B800 | 8x13 | 4 |
| 25 | APA | 640x480 | 16 | 80x60 | A000 | 8x8 | 1 |
| 26 | A/N | 720x480 | 16 | 80x60 | B800 | 9x8 | 3 |
| 29 | APA | 800x600 | 16 | 100x37 | A000 | 8x16 | 1 |
| 2A | A/N | 800x600 | 16 | 100x40 | B800 | 8x15 | 4 |
| 2D | APA | 640x350 | 256 | 80x25 | A000 | 8x14 | 1 |
| 2E | APA | 640x480 | 256 | 80x30 | A000 | 8x16 | 1 |
| 2F | APA | 640x400 | 256 | 80x25 | A000 | 8x16 | 1 |
| 30 | APA | 800x600 | 256 | 100x37 | A000 | 8x16 | 1 |
| 37 | APA | 1024x768 | 16 | 128x48 | A000 | 8x16 | 1 |
| 38 | APA | 1024x768 | 256 | 128x48 | A000 | 8x16 | 1 |
| 39 | APA | 1280x1024 | 16 | 160x64 | A000 | 8x16 | 1 |
| 3A | APA | 1280x1024 | 256 | 160x64 | A000 | 8x16 | 1 |
| 40 | APA | 320x200 | 32K | 40x25 | A000 | 8x8 | 1 |
| 41 | APA | 320x200 | 64K | 40x25 | A000 | 8x8 | 1 |
| 42 | APA | 320x200 | 16.8M | 40x25 | A000 | 8x8 | 1 |
| 43 | APA | 640x480 | 32K | 80x30 | A000 | 8x16 | 1 |
| 44 | APA | 640x480 | 64K | 80x30 | A000 | 8x16 | 1 |
| 45 | APA | 640x480 | 16.8M | 80x30 | A000 | 8x16 | 1 |
| 46 | APA | 800x600 | 32K | 100x37 | A000 | 8x16 | 1 |
| 47 | APA | 800x600 | 64K | 100x37 | A000 | 8x16 | 1 |
| 48 | APA | 800x600 | 16.8M | 100x37 | A000 | 8x16 | 1 |
| 49 | APA | 1024x768 | 32K | 128x48 | A000 | 8x16 | 1 |
| 4A | APA | 1024x768 | 64K | 128x48 | A000 | 8x16 | 1 |
| 4B | APA | 1024x768 | 16.8M | 128x48 | A000 | 8x16 | 1 |
| 4C | APA | 1280x1024 | 32K | 160x64 | A000 | 8x16 | 1 |
| 4D | APA | 1280x1024 | 64K | 160x64 | A000 | 8x16 | 1 |

Note:	1. A/N: Alpha/Numeric

2. APA: All Point Addressable (Graphic)

| MODE | DISPLAY SIZE | COLORS SHADES | FRAME RATE. | H-SYNC. | VIDEO FREQ. |
| --- | --- | --- | --- | --- | --- |
| 22 | 1056x352 | 16 | 70 | 30.5 K | 40.0 M |
| 23 | 1056x350 | 16 | 70 | 30.5 K | 40.0 M |
| 24 | 1056x364 | 16 | 70 | 30.5 K | 40.0 M |
| 25 | 640x480 | 16 | 60 | 31.5 K | 25.1 M |
| 26 | 720x480 | 16 | 60 | 31.5 K | 25.1 M |
| 29 | 800x600 | 16 | 56 | 35.1 K | 30.0 M |
| 29* | 800x600 | 16 | 60 | 37.9 K | 40.0 M |
| 29+ | 800x600 | 16 | 72 | 48.0 K | 50.0 M |
| 29# | 800x600 | 16 | 75 | 46.8 K | 50.0 M |
| 2A | 800x600 | 16 | 56 | 35.1 K | 36.0 M |
| 2D | 640x350 | 256 | 70 | 31.5 K | 25.1 M |
| 2E | 640x480 | 256 | 60 | 31.5 K | 25.1 M |
| 2E* | 640x480 | 256 | 72 | 37.9 K | 31.5 M |
| 2E+ | 640x480 | 256 | 75 | 37.5 K | 31.5 M |
| 2F | 640x400 | 256 | 70 | 31.5 K | 25.1 M |
| 30 | 800x600 | 256 | 56 | 35.1 K | 36.0 M |
| 30* | 800x600 | 256 | 60 | 37.9 K | 40.0 M |
| 30+ | 800x600 | 256 | 72 | 48.0 K | 50.0 M |
| 30# | 800x600 | 256 | 75 | 46.8 K | 50.0 M |
| 37i | 1024x768 | 16 | 87 | 35.5 K | 44.9 M |
| 37n | 1024x768 | 16 | 60 | 48.4 K | 65.0 M |
| 37n+ | 1024x768 | 16 | 70 | 56.5 K | 75.0 M |
| 37n# | 1024x768 | 16 | 75 | 60.2 K | 80.0 M |
| 38i | 1024x768 | 256 | 87 | 35.5 K | 44.9 M |
| 38n | 1024x768 | 256 | 60 | 48.4 K | 65.0 M |
| 38n+ | 1024x768 | 256 | 70 | 56.5 K | 75.0 M |
| 38n# | 1024x768 | 256 | 75 | 60.2 K | 80.0 M |
| 39i | 1280x1024 | 16 | 89 | 48.8 K | 80.0 M |
| 39n | 1280x1024 | 16 | 60 | 65.0 K | 110.0 M |
| 3Ai | 1280x1024 | 256 | 89 | 48.8 K | 80.0 M |
| 3An | 1280x1024 | 256 | 60 | 65.0 K | 110.0 M |

| 40 | 320x200 | 32K | 70 | 31.5 K | 25.1 M |
| --- | --- | --- | --- | --- | --- |
| 41 | 320x200 | 64K | 70 | 31.5 K | 25.1 M |
| 42 | 320x200 | 16.8M | 70 | 31.5 K | 25.1 M |
| 43 | 640x480 | 32K | 60 | 31.5 K | 25.1 M |
| 43* | 640x480 | 32K | 72 | 37.9 K | 31.5 M |
| 43+ | 640x480 | 32K | 75 | 37.5 K | 31.5 M |
| 44 | 640x480 | 64K | 60 | 31.5 K | 25.1 M |
| 44* | 640x480 | 64K | 72 | 37.9 K | 31.5 M |
| 44+ | 640x480 | 64K | 75 | 37.5 K | 31.5 M |
| 45 | 640x480 | 16.8M | 60 | 31.5 K | 25.1 M |
| 45* | 640x480 | 16.8M | 72 | 37.9 K | 31.5 M |
| 45+ | 640x480 | 16.8M | 75 | 37.5 K | 31.5 M |
| 46 | 800x600 | 32K | 56 | 35.1 K | 36.0 M |
| 46* | 800x600 | 32K | 60 | 37.9 K | 40.0 M |
| 46+ | 800x600 | 32K | 72 | 48.0 K | 50.0 M |
| 46# | 800x600 | 32K | 75 | 46.8 K | 50.0 M |
| 47 | 800x600 | 64K | 56 | 35.1 K | 36.0 M |
| 47* | 800x600 | 64K | 60 | 37.9 K | 40.0 M |
| 47+ | 800x600 | 64K | 72 | 48.0 K | 50.0 M |
| 47# | 800x600 | 64K | 75 | 46.8 K | 50.0 M |
| 48 | 800x600 | 16.8M | 56 | 35.1 K | 36.0 M |
| 48* | 800x600 | 16.8M | 60 | 37.9 K | 40.0 M |
| 48+ | 800x600 | 16.8M | 72 | 48.0 K | 50.0 M |
| 48# | 800x600 | 16.8M | 75 | 46.8 K | 50.0 M |
| 49i | 1024x768 | 32K | 87 | 35.5 K | 44.9 M |
| 49n | 1024x768 | 32K | 60 | 48.4 K | 65.0 M |
| 49n+ | 1024x768 | 32K | 70 | 56.5 K | 75.0 M |
| 49n# | 1024x768 | 32K | 75 | 60.2 K | 80.0 M |
| 4Ai | 1024x768 | 64K | 87 | 35.5 K | 44.9 M |
| 4An | 1024x768 | 64K | 60 | 48.4 K | 65.0 M |
| 4An+ | 1024x768 | 64K | 70 | 56.5 K | 75.0 M |
| 4An# | 1024x768 | 64K | 75 | 60.2 K | 80.0 M |
| 4Bi | 1024x768 | 16.8M | 87 | 35.5 K | 44.9 M |
| 4Ci | 1280x1024 | 32K | 89 | 48.8 K | 80.0 M |
| 4Di | 1280x1024 | 64K | 89 | 48.8 K | 80.0 M |

Note:	i - interlaced mode

n - noninterlaced mode

·	For the limitation of memory bandwidth in 1MB DRAM configuration, the following video modes is not supported in 1MB configuration:  modes 45*, 45+, 46+, 46#, 47+, and 47#.

# Chapter 4.

Software Drivers

To make use of the advance features of SiS 6202, extended graphic and text modes are supported by software application drivers developed by SiS.  The following applications are currently supported:

·	3D Studio Ver. 3.0

·	AutoCAD/386 Release 11, 12

·	Auto Shade/386 Ver. 2.0

·	GEM 3.0/Ventura 2.0

·	Lotus 1-2-3/Symphony Ver. 2.x

·	MicroSoft Windows 3.1

·	MicroSoft Windows NT Ver. 3.1 & 3.5

·	OrCad (SDT/VST/PCB) Rel 4

·	OS/2 Presentation Manager 2.1 & 3.0

·	P-CAD Ver. 6.06

·	VersaCAD/386 Ver. 2.1

·	Word Perfect 5.x & 6.0

# 1. Windows 3.1

## 1.1 Driver Files

1. 	The enclosed SiS 6202 Windows 3.1 driver contains the following files (in compressed format) :

Setup Programs		A group of programs used to setup drivers.

VGA800.DR_ 		800x600	16-color driver

VGA1024.DR_ 		1024x768	16-color driver

VGA1280.DR_ 		1280x1024	16-color driver

VGA256.DR_ 		256-color driver (for all resolution)

VGA32K.DR_ 		32K-color driver (for all resolution)

VGA64K.DR_ 		64K-color driver (for all resolution)

VGA16M.DR_ 		16M-color driver (for all resolution)

VDDSIS.386			Graber file for all resoution and color

OEMSETUP.INF		OEMSETUP file for setup procedure

OEMSIS.INF			Same content as OEMSETUP.INF for setup procedure

2. 	The amount of video memory required for each resolution are as follow:

| Resolution | 1 MByte DRAM | 2 MByte DRAM | 4 MByte DRAM |
| --- | --- | --- | --- |
| 640x480x8 | Ö | Ö | Ö |
| 640x480x16 | Ö | Ö | Ö |
| 640x480x24 | Ö | Ö | Ö |
| 800x600x4 | Ö | Ö | Ö |
| 800x600x8 | Ö | Ö | Ö |
| 800x600x16 | Ö | Ö | Ö |
| 800x600x24 | X | Ö | Ö |
| 1024x768x4 | Ö | Ö | Ö |
| 1024x768x8 | Ö | Ö | Ö |
| 1024x768x16 | X | Ö | Ö |
| 1024x768x24 | X | X | Ö |
| 1280x1024x4 | Ö | Ö | Ö |
| 1280x1024x8 | X | Ö | Ö |
| 1280x1024x16 | X | X | Ö |

Note:

1. 	x4 (4 bit/pixel): 	16-color mode,

x8 (8 bit/pixel): 	256-color mode,

x16 (16 bit/pixel):	64K-color mode,

x24 (24 bit/pixel):	16M-color mode.

2. 	For 32K-color mode,

it uses 15 bit/pixel (x15) and

requires the same amount of memory as 64K-color mode (x16).

3. 	If you setup a mode requires more video memory than you installed on your adapter,

the result is unpredictable.

## 1.2 Installation

### 1.2.1 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Boot up Windows using standard VGA mode.

2. 	Insert "SiS 6202 Drivers Disk #?" into drive A or drive B.

3. 	In Windows Program Manager Screen,

choose "File" item.

4. 	In "File Item List",

choose "Run" item.

5. 	In "Run" Screen,

key-in "a:\setup" or "b:\setup".

6. 	Follows the directions appeared on the screen to complete the unpack & copy procedures.

7. 	After unpack & copy complete,

a "SiS Multimedia Vx.xx" program group would be created and shown on screen.

### 1.2.2  Graphics Setup

1.	In "SiS Multimedia Vx.xx" program group,

choose "SiS Multimedia Package" icon to enter setup screen.

2. 	In setup screen,

choose which options you would like to use.

3.	After completing your selections,

choose "OK" to make all your selections effective.

4.	Choose "Restart Winodws"

to re-boot Windows using new settings.

Or, choose "Continue"

to continue your current Windows processes.

(But when you re-boot Windows, the new settings would take effect.)

### 1.2.3  Power Saving Setup in Windows

1. 	In setup screen,

choose "power_saver" item to enter power_saver setup screen.

2. 	In "Power_Saver Setup" Screen,

choose which options you would like to use.

3. 	After completing the selections,

choose "OK" to make all your selections effective.

4. 	After complete setup,

the power_saver would take effect as you request when time up and

you may continue your normal process.

### 1.2.4  Zoom_Key Setup

0. 	In setup screen, choose "zooming" item to define "hot keys" for zoom-in or zoom-out screen without entering setup program.

The operation principles of zoom-in and zoom-out are as follow:

(1)	The resolution change sequence for zoom-in is

1024x768   ===>   800x600   ===>   640x480   .

(2)	The resolution change sequence for zoom-out is

640x480   ===>   800x600   ===>   1024x768   .

(3)	But you must first be able to zoom-in before you may zoom-out,

that means you can not get a resolution larger than that you setup.

To use this feature, please follow the following procedures.

1. 	In setup screen,

choose "zooming" item to enter "Zooming Hotkey" screen.

2. 	In "Zooming Hotkey" Screen,

choose which "hot key" you would like to use and enable it.

3. 	After completing the selections,

choose "OK" to make all your selections effective.

4. 	After complete setup,

you may use your own defined hot key to zoom-in or zoom-out screen.

### 1.2.5 Notes

1. 	The setup programs should be placed on the root of the diskette and the drivers should be placed on the subdirectory "\windows".

2. 	All the driver files (i.e. "VGA*.DRV") are in compressed format, therefore you can't use Windows Setup Program to setup them before they are de-compressed.

3. 	The power saver's timer settings would be effective even exit Windows back to DOS.

# 2. OS/2 V3.0  (Warp)

## 2.0	Driver & OS/2 Version Notes

1.  	The following description apply to "SiS 6202 Driver Release V1.13" and maybe later release version but not guaranteed.

2.  	All the OS/2 Warp Versions up to SiS 6202 driver Rev. 1.13 would be installed as described in this section except for Full Pack Double Bytes OS/2 Warp (i.e. Chinese, Japanese, Korea, etc.).  (Full Pack means including Win-OS/2 programs.)

3.  	For Full Pack Double Bytes OS/2 Warp installation, please refer to "Sec.  12  Full Pack Double Bytes OS/2 Warp".

## 2.1	Driver Files

1. 	The SiS 6202 OS/2 3.0 driver files resides on the "SiS 6202 Driver Diskette #3 Sub-directory \OS2WARP\SBCS.30" for single byte OS/2 Warp and "SiS 6202 Driver Diskette #4 Subdirectory \OS230D\DBCS.30" for double byte OS/2 Warp.

2. 	The enclosed SiS 6202 OS/2 3.0 driver contains the following files:

SISINST.CMD 	SiS driver install program

SVGA.EXE 		SiS PMI Generator

S768256.DL@ 	SiS IBMDEV32.DLL Display Driver

OTHERS 		other files required during installation

## 2.2 Installation

Before install SiS 6202 OS/2 drivers, make sure:

1. 	Install your OS/2 system using "VGA display" option (i.e. standard VGA).

2. 	Start your OS/2 system.

Install SiS 6202 OS/2 Warp drivers as following procedures:

1.	Enter "OS/2 window" or "OS/2 full screen".

2. 	Insert "SiS 6202 Driver" diskette #3 or #4 in drive A or drive B.

3.	Change directory to which holds the SiS 6202 OS/2 3.0 display drivers and type

SISINST <Enter>.

For example,

A:\OS2WARP>SISINST

4.	All the Driver Files will be copied to a subdirectory C:\SISDRV and

the "Resolution and Frame Rate Selection" menu appears and

all the resolution (and color) and frame rate supported would be shown on the screen.

5. 	Choose which one you would like to use and click "OK".

6. 	Then installation program would complete all installation process and create a "SiS Setup" for future change mode usage.

7. 	Shutdown and re-boot OS/2 to make your selection effectively.

# 3. Autodesk ADI 4.2 -Protected Mode

## 3.0 General Description

### 3.0.1 Driver Files

1.	The enclosed SiS 6202 ADI driver contains the following file:

RCPSIS.EXP 	SiS ADI Driver (for all resolutions & colors)

Note: This version of ADI driver does not support 16-color operation.

2. 	This driver fits for a series of Autodesk Inc. products including:

(1) AutoCAD/386 R11

(2) AutoCAD/386 R12

(3) AutoShade/386 V2.0

(4) 3D Studio V3.0

3. 	Their installtion procedures are different from one program to the others.

But the first step of installation is the same for all these programs, that is

"To unpack and copy drivers to where you would like them to reside."

Therefore, we will state this step below.

4. 	As to the real installation procedures for each program, we will give a detail description in Sec. 31 to Sec. 3.4.

### 3.0.2 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "G. ADI 4.2" to unpack and copy drivers.

(To select, type "G".)

5. 	In "Unpack & Copy ADI 4.2 Drivers" screen,

keyin the "drive:\directory" where these drivers would reside

(default C:\ADI42).

Program would unpack & copy all related driver files to where you assign.

6. 	After "unpack and copy" completed,

exit the INSTDRV.EXE program.

7. 	Refer to Sec. 3.1 to Sec. 3.4 for the real installation procedures for each program.

## 3.1 AutoCAD R11 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your ADI 4.2 drivers are located in C:\ADI42.

2.	Add the following setting to your own batch file for AutoCAD R11 (say ACADR11.BAT) or to your "AUTOEXEC.BAT" file:

SET DSPADI=\ADI42\RCPSIS.EXP  <Enter>

3.	Delete the configure file ACAD.CFG resides in \ACAD directory.

4.	Type

ACADR11 	<Enter>

to configure your AutoCAD R11 system.

5.	In "Select Display Device:" item,

choose "ADI P386 V4.0/4.1 display.".

6. 	In "Select Display Resolution" screen,

choose which display driver you want to use.

7.	Go through the whole instructions, and the system would start with the desired display setting.

## 3.2 AutoCAD R12 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your SiS ADI 4.2 drivers are located in C:\ADI42.

(3) Your AutoCAD R12 program is located in C:\ACADR12.

(4) Your AutoCAD R12 default drivers are located in C:\ACADR12\DRV.

(5) Your AutoCAD R12 configure file ACAD.CFG is located in C:\ACADR12.

2. 	Copy the following driver file to C:\ACADR12\DRV:

RCPSIS.EXP

.

You may complete this step by

COPY C:\ADI42\RCPSIS.EXP C:\ACADR12\DRV

.

3. 	Delete your original ACAD.CFG file.

You may complete this step by

DEL C:\ACADR12\ACAD.CFG

.

4. 	Restart your AutoCAD R12 program as usual.

5. 	AutoCAD R12 will ask you to complete the configuration procedures

since it can't find the configure file ACAD.CFG.

6. 	Follow the instructions of AutoCAD R12 to proceed configuration.

7. 	In "Available Video Displays:" item,

choose the "SiS Super VGA ADI v4.2 Display and Rendering driver" item.

8. 	In "Select Display Resolution" screen,

choose which display driver you want to use.

9. 	Go through the whole instructions, and the system would start with the selected display setting.

## 3.3 AutoShade R2.0 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your ADI 4.2 drivers are located in C:\ADI42.

2. 	Add the following settings to your batch file for AutoShade R2.0 (say SHADE2.BAT) or to your "AUTOEXEC.BAT" file.

(a) For display driver setting,

SET DSPADI=\ADI42\RCPSIS.EXP

.

(b) For rendering driver setting,

SET RDPADI=\ADI42\RCPSIS.EXP

.

3. 	Delete the configure file SHADE.CFG.

4. 	Type

SHADE2 	<Enter>

to re-configure the AutoShade.

5. 	While prompting "Select display device:",

choose "P386 AutoDesk Device Interface display driver.".

6. 	While prompting "Select rendering display driver:",

choose "P386 AutoDesk Device Interface rendering driver.".

7. 	Go through all the instructions,

the system would start with the desired display setting.

## 3.4 3D Studio Version 3.0 Setup

1. 	The following procedures assume that

(1) You have complete "unpack & copy" procedure.

(2) Your ADI 4.2 drivers are located in C:\ADI42.

2. 	Create your own 3D Studio V3.0 batch file (say 3DS3.BAT) and add the following settings to it, or

add the following settings to your "AUTOEXEC.BAT" file.

(1) 	SET RCPADI=C:\ADI42\RCPSIS.EXP

(2) 	SET RDPADI=C:\ADI42\RCPSIS.EXP

3. 	Execute the new 3DS batch file or

reboot the computer using the new "AUTOEXEC.BAT"

as to make the new settings effective.

4. 	Change your current working directory to \3DS3

(where your 3D Studio V3.0 usually resides).

5. 	Delete original configuration file "3DADI.CFG".

6. 	Type

3DS VIBCGF	<Enter>

to configure your display environment.

7. 	After the "Company Register Screen" appears,

press		<Enter>

to continue.

8. 	The "Video Environment Configuration Screen" will appear.

Please follow the following procedures to configure your video display environment.

(1)	In "Main-Display" item,

(a)	Press		<Enter>

The selection menu will appear.

(b)	In selection menu,

move cursor to "RCPADI".

Press		<Enter>

to select.

(2)	In "Material-Display" item,

(a)	Press		<Enter>

The selection menu will appear.

(b)	In selection menu,

move cursor to "RCPADI".

Press		<Enter>

to select.

(3)	In "Render-Display" item,

(a)	Press		<Enter>

The selection menu will appear.

(b)	In selection menu,

move cursor to "RCPADI" or "RDPADI".

Press		<Enter>

to select.

(4) 	Complete the other selections and exit configuration.

9. 	After exit configure,

3DS will boot automatically

using the environment you just select.

10. 	If your previous configuration is OK,

3DS will ask you to make detail configuration for SiS 6202 drivers.

If this didn't happen,

please check your previous procedures or contact the technical support people.

11. 	In the detail configuration for SiS 6202 drivers,

just follow the instructions appear on the screen and make your own choice.

If you are confused in anything, contact the technical support people for solution.

12. 	After detail configuration,

you will enter the 3DS main display screen and

you may begin your 3D Studio work

in the environment you make before.

13. 	Once completing the detail configuration,

you may enter 3D Studio in the same configuration simply by type

\3DS3\3DS 	<Enter>

next time.

14. 	If you want to change your video configuration,

just follow the procedures mentioned before to re-configure.

# 4. WordPerfect V5.1

## 4.1 Driver File

1. 	The enclosed SiS 6202 WordPerfect driver (packed file) is

"WP51.1".

2.	Being unpacked, the SiS 6202 WordPerfect driver "WPSIS.VRS" supports

Graphic Mode Resolution

640x350 	16 Colors

640x480 	16 Colors

800x600 	16 Colors

1024x768 	16 Colors

1280x1024 16 Colors

Text Mode Resolution

132x25

132x28

132x44

3. 	Use "INSTDRV.EXE" to

(1) unpack and copy drivers to where they should resides,

## 4.2 Installation

### 4.2.1 Unpack & Copy

To unpack and copy driver to where it should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "A. WordPerfect 5.1" to unpack and copy driver.

(To select, type "A".)

4. 	In "Unpack & Copy WordPerfect 5.1 Drivers" screen,

keyin the "drive:\directory" where your WordPerfect program resides.

Program would unpack & copy all related driver files to where you assign.

5. 	After "unpack and copy" completed,

exit the INSTDRV.EXE program.

### 4.2.2 WordPerfect Setup

1.	Run WordPerfect.

2.	From the Main menu,

choose File menu.

3.	From the File menu,

choose Setup menu.

4. 	From the Setup menu,

choose Display menu.

5. 	For graphic mode configuration,

(1) 	From the "Setup: Display" menu,

choose "Graphics Screen Type".

(2) 	From the "Setup: Graphics Driver" menu,

choose "SiS 6202 SVGA (16 Color)"

(3) 	From the "Setup: Graphics Driver" menu,

choose one of the following:

SiS 1024x768 16 Color

SiS 1280x1024 16 Color

SiS 640x350 16 Color

SiS 640x480 16 Color

SiS 800x600 16 Color

6. 	for text mode configuration,

(1) 	From the "Setup: Display" menu,

choose "Text Screen Type",

(2)	From the "Setup: Text Driver" menu

choose "SiS 6202 SVGA"

(3)	From the "Setup: Text Driver" menu,

choose from

SiS 132x25 16 Color Save Font

SiS 132x28 16 Color Save Font

SiS 132x44 16 Color Save Font

7. 	Exit from the menus and restart WordPerfect.

# 5. GEM 3.0 / Ventura 2.0

## 5.1 Driver Files

1. 	The enclosed SiS 6202 GEM / Ventura drivers (packed files) are

(1) GEMDRV.1

(2) GEMDRV.2

2. 	Being unpacked, the SiS 6202 GEM / Ventura drivers contain the following files:

(1) SDFSIS86.VGA: 		800x600 	16-color

(2) SDFSIS1K.VGA: 	1024x768 	16-color

3. 	Use "INSTDRV.EXE" to

(1) unpack and copy drivers to where they should resides,

(2) create new VP.BAT file.

## 5.2 Installation

### 5.2.1 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "B. GEM 3.0 / Ventura 2.0" to unpack, copy, and setup drivers.

(To select, type "B".)

4. 	In "Install and Setup GEM 3.0 / Ventura 2.0 Drivers" menu,

select "A. Unpack and Copy Drivers" to unpack and copy drivers.

(To select, type "A".)

5. 	In "Unpack & Copy GEM 3.0/Ventura 2.0 Drivers" screen,

keyin the "drive:\directory" where your GEM/Ventura program resides.

Program would unpack & copy all related driver files to where you assign.

6. 	After "unpack and copy" completed,

back to "Install and Setup GEM 3.0 / Ventura 2.0 Drivers" menu.

7. 	If you want to make one of the drivers effective now,

follow the Sec. 5.2.2 procedures to do it.

Otherwise you may exit INSTDRV.EXE program,

but the SiS 6202 driver won't be effective.

### 5.2.2 Ventura Setup --- Create New VP.BAT

To create new VP.BAT,

please follow the following procedures:

1. 	In "Install and Setup GEM 3.0 / Ventura 2.0 Drivers" menu,

select "B. Setup Ventura Batch File VP.BAT"

to create new Ventura batch file VP.BAT using new driver.

(To select, type "B".)

2. 	Fom the "Setup Ventura Batch File" screen,

keyin the drive letter for the hard disk where Ventura software resides.

3. 	From the "Setup Ventura Batch File" menu,

choose from

800x600 	16 Colors

1024x768 	16 Colors

4. 	After the message

"Are you sure (Y/N) ?"

appears on the screen,

keyin

'Y'  <Enter>.

5. 	After setup completes,

exit INSTDRV.EXE program.

6. 	Use new "VP.BAT" to start your Ventura progarm.

# 6. PCAD 6.06

## 6.1 Driver Files

1. 	The enclosed SiS 6202 PCAD drivers (packed files) are

(1) PCAD.1

(2) PCAD.2

(3) PCAD.3

2. 	Being unpacked, the SiS 6202 PCAD drivers contain the following files:

(1) PSIS800.REX: 		800x600 	16 Colors

(2) PSIS1K.REX: 		1024x768 	16 Colors

(3) PSIS12.REX: 		1280x1024 	16 Colors

3. 	Use "INSTDRV.EXE" to

(1) unpack and copy drivers to where they should resides,

(2) create new PCADDRV.SYS file.

## 6.2 Installation

### 6.2.1 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "C. PCAD 6.06" to unpack & copy and setup drivers.

(To select, type "C".)

4. 	In "Install and Setup PCAD 6.06 Drivers" menu,

select "A. Unpack and Copy Drivers" to unpack and copy drivers.

(To select, type "A".)

5. 	In "Unpack & Copy PCAD 6.06 Drivers" screen,

keyin the "drive:\directory" where these drivers would reside.

Program would unpack & copy all related driver files to where you assign.

6. 	After "unpack and copy" completed,

back to "Install and Setup PCAD 6.06 Drivers" menu.

7. 	If you want to make one of the drivers effective now,

follow the Sec. 6.2.2 procedures to do it.

Otherwise you may exit INSTDRV.EXE program,

but the SiS 6202 driver won't be effective.

### 6.2.2 PCAD Setup --- Create New PCADDRV.SYS

To create new PCADDRV.SYS, please follow the following procedures:

1. 	In "Install and Setup PCAD 6.06 Drivers" menu,

select "B. Setup Configure File PCADDRV.SYS"

to create new PCADDRV.SYS using new driver.

(To select, type "B".)

2. 	From the "Setup PCAD Config File" screen,

keyin the drive letter of the hard disk where PCAD software resides.

3. 	From the "Setup PCAD Config File" screen,

keyin the directory where the PCAD drivers reside.

4. 	From the "Setup PCAD Config File" menu,

choose from

800x600 	16 Colors

1024x768 	16 Colors

1280x1024 	16 Colors

5. 	After the message

"Are you sure (Y/N) ?"

appears on the screen,

keyin

'Y' 	<Enter>

.

6. 	After setup completes,

exit INSTDRV.EXE program.

7. 	Restart your PCAD program.

# 7. VersaCAD/386 2.1

## 7.1 Driver Files

1. 	The enclosed SiS 6202 VersaCAD/386 drivers (packed files) are

(1) VCAD.1

(2) VCAD.E

2. 	Being unpacked, the SiS 6202 VersaCAD/386 driver "VSIS.EXE" supports

640x480 	16 Colors

800x600 	16 Colors

1024x768 	16 Colors

1280x1024 	16 Colors

3. 	This driver is a TSR (Terminate and Stay Resident) type program.

You just need to execute once each time your computer is powered up.

For this reason, it is recommended that

this driver should be placed in your "AUTOEXEC.BAT" file.

4. 	Use "INSTDRV.EXE" to

(1) unpack and copy driver VSIS.EXE

(2) unpack and copy configure file VSIS.CFG

to where they should resides

## 7.2 Installation

### 7.2.1 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "D. VersaCAD/386 V2.1" to unpack and copy drivers.

(To select, type "D".)

4. 	In "Unpack and Copy VersaCAD/386 2.1 Drivers" menu,

keyin the "drive:\directory" where your VersaCAD/386 program resides.

Program will unpack and copy

(1) driver file "VSIS.EXE" and

(2) configure file "VSIS.CFG"

to where you keyin.

5. 	After "unpack and copy" complete,

exit INSTDRV.EXE program.

### 7.2.2 VersaCAD/386 Setup

1. 	Change the current "drive:\directory" path to where VersaCAD/386 resides.

2. 	Run this driver VSIS.EXE.

For example:

C:\VCAD386>VSIS 	<Enter>

3. 	Run ENVIRO program.

For example:

C:\VCAD386>ENVIRO 	<Enter>

4. 	From "ENVIRONMENT MENU" screen,

press 'S' key to select screen configuration.

5. 	In prompt mode, select "SiS 6202" display driver.

You can select various display resolution.

6. 	After selecting screen configuration,

press 'X' key to save configuration

7. 	Exit ENVIRO program.

8. 	Restart your VersaCAD program.

# 8. OrCAD Rel. 4

## 8.1 Driver Files

1. 	The enclosed SiS 6202 OrCAD drivers (packed files) are

(1) ORCAD.1

(2) ORCAD.2

(3) ORCAD.3

2. 	Being unpacked, the SiS 6202 OrCAD drivers contain the following files:

(1) ORSIS86.DRV: 	800x600 	16 Colors

(2) ORSIS1K.DRV: 	1024x768 	16 Colors

(3) ORSIS12.DRV: 	1280x1024 	16 Colors

3. 	Use "INSTDRV.EXE" to

(1) unpack and copy drivers to where they should resides,

(2) create new SDT.CFG file.

## 8.2 Installation

### 8.2.1 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "E. OrCAD rel 4" to unpack and copy drivers.

(To select, type "E".)

4. 	In "Install and Setup OrCAD rel 4 Drivers" menu,

select "A. Unpack and Copy Drivers" to unpack and copy drivers.

(To select, type "A".)

5. 	In "Unpack & Copy OrCAD rel 4 Drivers" screen,

keyin the "drive:\directory" where these drivers should reside.

(default C:\ORCADESP\DRV)

Program would unpack & copy all related driver files to where you assign.

6. 	After "unpack and copy" completed,

back to "Install and Setup OrCAD rel 4 Drivers" menu.

7. 	If you want to create new SDT.CFG,

follow the Sec. 8.2.2 procedures to do it.

Otherwise you may exit INSTDRV.EXE program,

but the SiS 6202 driver won't be effective.

### 8.2.2 OrCAD Display Setup

To setup SiS display driver for OrCAD,

please follow the following procedures:

1. 	Enter OrCAD "Main Menu" screen.

2. 	For "Schematic Tools Configure",

choose "Schematic Design Tools"

to enter "Schematic Design Tools" screen.

3. 	In "Schmatic Design Tools" screen,

choose "draft" block.

4. 	"Draft Menu" will appear,

choose "Configure Schematic Tools" item

to configure schematic tools.

5. 	"Configure Schematic Tools" screen will appear,

choose "Driver Prefix" item and

keyin where SiS OrCAD drivers reside (e.g. \ORCADESP\DRV\).

6. 	In the same "Configure Schematic Tools" screen,

choose "Configured Display Driver" item and

keyin one of the SiS OrCAD drivers' name which you would like to use.

7. 	If you want to configure anything else,

do as you like.

8. 	When you complete all the configuration,

choose "OK" item

to ask OrCAD program to save your configuration.

9. 	After configuration completes,

you may use your OrCAD schematic tools in the configuration you just setup.

10. 	For all other tools configuration,

please follow the Step 1 to the Step 9 stated before.

Note:

1.	If you install your OrCAD program using default settings,

it's reommended you should use default settings

when you install SiS OrCAD drivers.

### 8.2.3 OrCAD SDT.CFG Setup

To create new SDT.CFG file,

please follow the following procedures:

1. 	In "Install and Setup OrCAD rel 4 Drivers" menu,

select "B. Setup Configure File SDT.CFG"

to create new SDT.CFG using new driver.

(To select, type "B".)

2. 	From the "Setup OrCAD r4 Driver Config File" screen,

keyin the "drive:\directory" where your OrCAD program resides

(default C:\ORCADEXE).

3. 	From the "Setup OrCAD r4 Driver Config File" screen,

keyin the "drive:\directory" where the SiS OrCAD drivers reside

(default C:\ORCADESP\DRV).

4. 	From the "Setup OrCAD Config File" menu, choose from

800x600 	16 Colors

1024x768 	16 Colors

1280x1024 	16 Colors

5. 	After the message

"Are you sure (Y/N) ?"

appears on the screen,

keyin

'Y' 	<Enter>

.

6. 	After setup completes,

exit INSTDRV.EXE program.

7. 	Restart your OrCAD DRAFT.EXE program.

Note:

1.	If you install your OrCAD program using default settings,

it's reommended you should use default settings

when you install SiS OrCAD drivers.

# 9. Lotus 2.x

## 9.1 Driver Files

1. 	The enclosed SiS 6202 Lotus 1-2-3 drivers (packed files) are

(1) LOTUS.1

(2) LOTUS.2

(3) LOTUS.3

(4) LOTUS.4

2. 	Being unpacked, the SiS 6202 Lotus 1-2-3 drivers contain the following files:

(1) TXT13225.DRV: 	132x25 Text

(2) TXT13228.DRV: 	132x28 Text

(3) TXT13244.DRV: 	132x44 Text

(4) TXT8060.DRV: 	80x60   Text

3. 	Use "INSTDRV.EXE" to

(1) unpack and copy drivers to where they should resides.

## 9.2 Installation

### 9.2.1 Unpack & Copy

To unpack and copy drivers to where they should resides,

please follow the following procedures:

1. 	Insert "Driver Setup" diskette in drive A or drive B.

2. 	Run "INSTDRV.EXE" program on drive A or drive B.

For example,

A:\>INSTDRV	<Enter>

3. 	In "SiS 6202 Super VGA Drivers Installation" menu,

select "F. Lotus 1-2-3 2.x" to unpack and copy drivers.

(To select, type "F".)

4. 	In "Unpack & Copy Lotus Drivers" screen,

keyin the "drive:\directory" where your Lotus program resides

(default C:\123).

Program would unpack & copy all related driver files to where you assign.

5. 	After "unpack and copy" completed,

exit INSTDRV.EXE program.

### 9.2.2 Lotus 1-2-3 Setup

1. 	Change the current "drive:\directory" path to where Lotus 1-2-3 software resides.

2. 	Run Lotus 1-2-3 install program.

For example:

C:\123>install 	<Enter>

3. 	Several menu choices will be presented on screen.

Follow the instructions to add SiS drivers to the library:

(1) 	Press <Enter> key to begin.

(2) 	From the main menu, choose "Advanced Options".

(3) 	From the "Advanced Options" menu,

choose "Add New Drivers to the Library".

(4) 	Press <Enter> key twice

to return to the "Advanced Options" menu.

4. 	After returning to "Advanced Options" menu,

follow the instructions to select the desired driver.

(1) 	Choose "Modify Current Driver Set" option.

(2) 	From the configuration menu,

choose "Text Display" option.

(3) 	From the "Text Display" menu,

select the desired driver.

(4)	 After returning to configuration menu,

choose "Return to menu".

5. 	After returning to "Advanced Options" menu,

follow the instructions to exit the INSTALL program.

(1) 	Choose "Save Changes" to save the changed configuration.

(2) 	Press RETURN key to accept the current set name.

(3) 	Press RETURN key again.

(4) 	From "Exit" menu,

choose "Yes" option to terminate the INSTALL program.

6. 	Restart your Lotus 1-2-3 program.

# 10. Windows NT 3.1 & 3.5

## 10.1 Driver Files

1. 	The enclosed SiS 6202 Windows NT 3.1 drivers are:

SISTAG

SISV.SYS

SISV256.DLL

SISV.DLL

OEMSETUP.INF

2. 	At present only 16-color and 256-color drivers are available.

32K/64K-color and 16M-color drivers are under development.

3. 	There is no "Unpack & Copy" procedure in Windows NT 3.1 driver installation.

## 10.2 Installation

### 10.2.1 Windows NT Setup

1. 	Boot up Windows NT.

2. 	Run the following procedures:

a) Windows NT setup

b) Options

c) Change Systems Settings

d) Display

e) Other

Then respond to installation prompts.

# 11.  Windows 95

## 11.0  Version Note

The following description apply to "SiS 6202 Driver Release V1.12" and maybe later release version but not guaranteed.

## 11.1  Driver Files

1. 	The enclosed SiS 6202 Windows 95 drivers files are:

SISMINI.VXD

SIS202.INF

SIS202.DRV

Other Files

2. 	All the 16-color, 256-color, 32K/64K-color, and 16M-color drivers are available.

3. 	There is no "Unpack & Copy" procedure in Windows 95 driver installation.  You have to follow the installation procedure to complete it.

4. 	The "SISTOOL.EXE" used in earlier version has been taken away and add an "SiS Settings" item in Windows 95 "Control Panel:  Display" for changing display options.

## 11.2  Installation and Using "SiS Settings"

1. 	Boot up Windows 95.

2. 	In Windows 95 "Booted" Screen,

select "Start" item.

3. 	In "Start" Screen,

select "Settings" item.

4. 	In "Settings" Screen,

select "Control Panel" item.

5. 	In "Control Panel" Screen,

select "Display" icon.

6. 	In "Display Properties" Screen,

select "Settings" item.

7. 	In refreshed "Display Properties" Screen,

select "Change Display_Type" item.

8. 	In "Change Display Type" Screen,

select "Adapter Type  ==>  Change" item.

9. 	In "Select Device" Screen,

select "Have Disk..." item.

10. 	In "Install From Disk" Screen,

select your driver source.

e.g.  	C:\SIS6202

11. 	In new "Select Device" Screen,

"SiS 6202" would appear and

select "OK".

Then Windows 95 would perform driver installation.

12. 	After completing installation,

Windows 95 would go back to "Change Display Type" Screen,

select "Close" item.

13. 	Windows 95 would go back to "Display Properties" Screen and

you will find that "SiS Settings" item had been added.

14. 	First select "SiS Settings" item and you may select

desired resolution by changing "Desktop area" pointer,

desired color by changing "Color palette" value,

desired font size by changing "Font size", and

desired refresh rate by changing "Refresh rate".

15. 	After completing selection,

choose "Apply" to complete the whole installation.

16. 	Then Windows 95 would ask

"Do you want to restart Windows?".

Choose "Restart" to make your selection active and

you may work at your selection mode.

# 12.  Full Pack Double Bytes OS/2 Warp

## 12.0  Driver & OS/2 Version Notes

1.  	The following description apply to "SiS 6202 Driver Release V1.13" and maybe later release version but not guaranteed.

2.  	The following description apply to "Double Bytes OS/2 Warp" only and especially design for "Full Pack Double Bytes OS/2 Warp".  (Full Pack means including Win-OS/2 programs.)

3.  	For other versions OS/2 Warp installation, please refer to "Sec. 2  OS/2 V3.0 (Warp)".

## 12.1	Driver Files

1. 	The SiS 6202 Double Byte OS/2 3.0 driver files resides on the "SiS 6202 Driver Diskette #4 Sub-directory OS230D\DBCS.30  (i.e. \OS230D\DBCS.30)".

2. 	The enclosed SiS 6202 Double Byte OS/2 3.0 driver contains the following files:

SETUP.CMD 	SiS driver install program

SVGA.EXE 		SiS PMI Generator

S768256.DL@ 	SiS IBMDEV32.DLL Display Driver

OTHERS 		other files required during installation

## 12.2 Installation

For Full Pack Double Byte OS/2 Warp limitation, the installation process is a two-phase installation and can't complete the whole installation in one-time-process.

1st phase:  	copy files, modify "config.sys", create "SiS Install" icon

then shutdown and re-boot OS/2.

2nd phase:  	double clicks "SiS Install" icon to complete the installation

Before install SiS 6202 OS/2 drivers, make sure:

1. 	Install your OS/2 system using "VGA display" option (i.e. standard VGA).

2. 	Start your OS/2 system.

Install SiS 6202 OS/2 Warp drivers as following procedures:

1st phase:

1.	Enter "OS/2 window" or "OS/2 full screen".

2. 	Insert "SiS 6202 Driver" diskette #4 in drive A or drive B.

3.	Change directory to which holds the SiS 6202 OS/2 3.0 display drivers and type

SETUP [drive] <Enter>, (where [drive] is the boot drive character).

For example,

A:\OS230D\DBCS.30>SETUP

4.	All the Driver Files will be copied to a subdirectory C:\SISDRV and "SiS Install" icon would be created.

5.	After completing "setup" program, shutdown and re-boot OS/2.

2nd phase:

6.	After re-booting OS/2, double clicks "SiS Install" icon.

Then the "SiS Install" icon would be executed automatically and "Resolution and Frame Rate Selection" screen would appeared on the screen.

7.	Select the desired resolution (and color) and frame rate on this screen,

then click "OK" to complete installation and "SiS Setup" icon would be created for future change modes and frame rate usage.

8.	After completing installation, shutdown and re-boot OS/2 to make your selection effectively.

# COPYRIGHT NOTICE

COPYRIGHT 1996, Silicon Integrated Systems Corp. ALL RIGHTS RESERVED.

This manual is copyrighted By Silicon Integrated Systems Corp.  You may not reproduce, transmit, transcribe, store in a retrieval system, or translate into any language, in any form or by any means, electronic, mechanical, magnetic, optical, chemical, manual, or otherwise, any part of this publication without the express written permission of Silicon Integrated Systems Corp.

TRADEMARKS

SiS is a registered trademark of Silicon Integrated Systems Corp.

All brand or product names mentioned are trademarks or registered trademarks of their respective holders.

DISCLAIMER

Silicon Integrated Systems Corp. makes no representations or warranties regarding the contents of this manual.  We reserve the right to revise the manual or make changes in the specifications of the product described within it at any time without notice and without obligation to notify any person of such revision or change.

The information contained in this manual is provided for general use by our customers. Our customers should be aware that the personal computer field ins the subject of many patents. Our customers should ensure that they take appropriate action so that their use of our products does not infringe upon any patents. It is the policy of Silicon Integrated Systems Corp. To respect the valid patent rights of third parties and not to infringe upon or assist others to infringe upon such rights.

RESTRICTED RIGHTS LEGEND

Use, duplication, or disclosure by the Government is subject to restrictions set forth in subparagraph (c)(1)(ii) of the Rights in Technical Data and Computer Software clause at 252.277-7013.