SVGAUTL.EXE is one of the utilities of SiS 6202 display cards.

It support three functions--Video Mode Setting, Frame Rate Setting, and

Power Saving Setting.

Since SiS 6202 support many enhanced Text Mode and Graphic Mode, you

can use SVGAUTL.EXE to select the desired video mode. For 640x480, 800x600,

1024x768 and 1280x1024 resolutions, 6202 supports multiple frame rates.

If your monitor could synchronize with these frame rates, then you can use

SVGAUTL.EXE to get advantage. By the way, 6202 also supports Power Saving,

SVGAUTL.EXE can help you to set the capability.

There are two syntax for SVGAUTL.EXE. One is as following example:

> SVGAUTL <ENTER>

then the Menu would appear and direct you to configure 6202.

The other syntax is as following :

> SVGAUTL [/D:mode_no] [/F0:n0] [/F1:n1] [/F2:n2] [/F3:n3] [/PA:ta] [/PB:tb]

where

/D    : Set the Video Mode to be mode_no which is a hex number.

For example : Set 1024x768 256 color graphic mode.

>SVGAUTL /D:38 <ENTER>

/F0   : For 640x480, set frame rate to be n0 Hz. Three availabe frame

rates are 60, 72 and 75 Hz.

For example : Set 640x480 graphic mode with 60Hz frame rate

>SVGAUTL /F0:60 <ENTER>

/F1   : For 800x600, set frame rate to be n1 Hz. Four availabe frame

rates are 56, 60, 72 and 75 Hz.

For example : Set 800x600 graphic mode with 72Hz frame rate.

>SVGAUTL /F1:72 <ENTER>

/F2   : For 1024x768, set frame rate to be n2 Hz. Four availabe frame

rates are 87, 60, 70 and 75 Hz.

For example : Set 1024x768 graphic mode with 60Hz frame rate.

>SVGAUTL /F2:60 <ENTER>

/F3   : For 1280x1024, set frame rate to be n3 Hz. Two availabe frame

rates are 87 and 60 Hz.

For example : Set 1280x1024 graphic mode with 60Hz frame rate.

>SVGAUTL /F3:60 <ENTER>

/PA   : Set Standby Timer to be ta*2 minutes.

For example : Set Standby Timer be 10 minutes.

>SVGAUTL /PA:5 <ENTER>

/PB   : Set Suspend Timer to be tb minutes.

For example : Set Suspend Timer be 10 minutes.

>SVGAUTL /PB:5 <ENTER>

If you want your PC booted by the display configuration setting, add your

setting to the AUTOEXEC.BAT file.