If Sony MS-DOS CD-ROM Extensions does not run well, check your hardware and

software.   This document describes the check sequence.

Make a hard copy of this file using PRINT command for your convenience.

Related documents.

1. Installation Manual of Sony MS-DOS CD-ROM Extensions.

2. Operating Instructions of each CD-ROM drive.

3. Operating Instructions of each host adapter for CD-ROM drive.

4. README.DOC file on this Library disk.

5. Technical reference manual of your computer.

6. Disk Operating manual of your operating system.

7. Other manuals of other peripherals.

NOTICE: MS-DOS 5.0 ignores MSCDEX.EXE as Incorrect DOS version.

Revision 2.20                       SONY Corporation

June, 21,1991                       CD-ROM Division,  Storage Systems Group

(c) Copyright 1988 - 1991, Sony Corporation.

MS-DOS CD-ROM Extensions is a registered trademark of Microsoft Corporation.

IBM is a registered trademark of International Business Machines Corporation.

###page  1

************************ TABLE OF CONTENTS ****************************

CHAPTER NUMBER AND THE TITLE                                              PAGE

CHAPTER 1. HARDWARE CHECK  ...............................................  2

1-1. HOST ADAPTER   ...................................................  2

1-1-1 ON-SCREEN IMAGE OF CDB-231  ..................................  3

1-1-2 ON-SCREEN IMAGE OF CDB-230. ..................................  4

1-1-3 ON-SCREEN IMAGE OF CDB-220 / 200  ............................  5

1-2. CONNECTION AND TERMINATORS   .....................................  6

1-3. SWITCH SETTINGS OF THE DRIVE(S)  ................................. 10

1-3-1. SELECT SWITCHES   ........................................... 10

1-3-2. DRQ/WAIT SWITCH   ........................................... 10

CHAPTER 2. SOFTWARE CHECK   .............................................. 11

2-1. BASE ADDRESS   ................................................... 11

2-2. IRQ CHANNEL   .................................................... 12

2-3. DMA CHANNEL   .................................................... 14

2-4. EXECUTABLE PATH AND SUB-DIRECTORY OF THE DEVICE DRIVER   ......... 15

2-5. DEVICE NAME   .................................................... 15

CHAPTER 3. ENVIRONMENT VARIABLE  AND DISK SPACE   ........................ 16

END OF CHCKLIST.DOC FILE. ................................................ 16

#

page  2

CHAPTER 1. HARDWARE CHECK.

1-1. HOST ADAPTER.

Check your host adapter first, by typing "CDB_SW" followed by Enter.

On-screen image of host adaptor card appear.

###page  3

1-1-1. ON-SCREEN IMAGE OF CDB-231.

+-------------------------------------+

|                                     |     Select the

|                                     |     Base Address to 340.

| Base address in HEX                 |

|                                     |

|                                     |

| 12345678   2345    IRQ CHANNEL      |     Set IRQ channel to

| ####+#++   ++++++                   |     all off.

| ++++#+##   ######                   |

|                                     |

|            123123  DMA CHANNEL      |     Set DMA channel to

|            ++++++                   |     all off.

|            ######                   |

+---------------------+            +--+

|            |

+------------+

# IS SWITCH POSITION.

#

page  4

1-1-2 ON-SCREEN IMAGE OF CDB-230.

+-------------------------------------+

|                                     |     Select the

|                                     |     Base Address to 340.

| Base address in HEX                 |

|                                     |

|                                     |

| 12345678   2456    IRQ CHANNEL      |   Set IRQ channel to

| ####+#++   ++++++                   |   all off.

| ++++#+##   ######                   |

|                                     |

|            123123  DMA CHANNEL      |   Set DMA channel to

|            ++++++                   |   all off.

|            ######                   |

+---------------------+            +--+

|            |

+------------+

# IS SWITCH POSITION.

###page  5

1-1-3 ON-SCREEN IMAGE OF CDB-220 / 200.

+-------------------------------------+

|                                     |     Select the

|                                     |     Base Address to 340.

| Base address in HEX                 |

|                                     |

|                                     |

| 12345678   123123  DMA CHANNEL      |   Set DMA channel to

| ####+#++   ++++++                   |   all off.

| ++++#+##   ######                   |

|                                     |

|            2456    IRQ CHANNEL      |   Set IRQ channel to

|            ++++++                   |   all off.

|            ######                   |

+---------------------+            +--+

|            |

+------------+

# IS SWITCH POSITION.

#

page  6

1-2. CONNECTION AND TERMINATORS.

Check connection and terminator settings of the drive and the host

adapter.

You can connect up to four CD-ROM drives in a chain.   The following pages

give you some typical examples of the terminator settings regarding the number

of connected drive(s).

One naked drive installed inside of the computer.  ==> page 7.

One stand alone drive outside of the computer.  ==> page 8.

One naked drive installed inside of the computer and

one stand alone drive outside of the computer.  ==> page 9.

###page  7

ONE NAKED DRIVE IN THE COMPUTER.

HOST COMPUTER

_____________________________________

|_______________                      |

|               |          CDB-220    |

| FDD           |          CDB-230    |

|_______________|          CDB-231    |

|_______________        ______________|

| CD-ROM drive  |      |              |

| Terminator ON |<=============>|| ** |

|_______________|      |____         _|

|___________________________|_______|_|

The "**" indicates the terminator resistors.

If your host adapter is CDB-231, never remove the terminator resistors.

The terminator switches of the drive should be set to on.

#

page  8

ONE STAND ALONE DRIVE.

HOST COMPUTER

_____________________________________

|_______________                      |

|               |       CDB-200       |

| FDD           |       CDB-230       |

|_______________|       CDB-231       |              _________________

|                       ______________|             |                 |

|                      |              |             |  CD-ROM drive   |

|                      |          **  |<===========>|                 |

|                      |____         _|             |  Terminator ON  |

|___________________________|_______|_|             |_________________|

The "**" indicates the terminator resistors.

If your host adapter is CDB-231, never remove the terminator resistors of

CDB-231.  The terminator switch of the drive should be set to on.

###page  9

ONE NAKED DRIVE IN THE COMPUTER AND ONE STAND ALONE DRIVE.

HOST COMPUTER

_____________________________________

|_______________                      |

|               |                     |

| FDD           |                     |

|_______________|                     |              _________________

|_______________        ______________|             |                 |

| CD-ROM drive  |      |   CDB-231    |             |  CD-ROM drive   |

|    #0         |<=============>|| ** |<===========>|    #1           |

|_Terminator ON_|      |____         _|             | Terminator ON   |

|___________________________|_______|_|             |_________________|

Remove the terminators.

The "**" indicates the terminator resistors.

Remove the terminator resistors from the sockets of CDB-231.

The terminator switches of the #0 and #1 should be set to on.

#

page 10

1-3. SWITCH SETTINGS OF THE DRIVE(S).

1-3-1. SELECT SWITCHES.

Check if the select switch settings of each drive is correct.

Most of case, the switch "0" is set to on and others are set to off

because only one CD-ROM drive is connected to the host adapter.   In this case

the drive letter which is specified in AUTOEXEC.BAT file is applied to the

drive.

If you have connected multiple drives in a chain, you have to set the

switches to different number for each drive.   Refer to Operating Instructions

of each CD-ROM drive.

Refer to CUSTOMIZATION of README.DOC file for the drive letter in

AUTOEXEC.BAT file.

1-3-2. DRQ/WAIT SWITCH.

You must set this switch to DRQ MODE.  Otherwise data error may happen.

###page 11

CHAPTER 2. SOFTWARE CHECK.

2-1. BASE ADDRESS.

If you have installed Sony MS-DOS CD-ROM Extensions using INSTALL.BAT,

the base address is set to 340h in CONFIG.SYS file.

Check the switch settings of the host adapter and of CONFIG.SYS file are same.

If not, you must set them to correct value and boot again.

To avoid conflict between MS-DOS CD-ROM Extensions and other peripherals, be

aware of the port addresses which are used by all peripherals installed on your

computer.  When host adapter(s) other than CDB-200, 220, 230 nor 231 is (are)

installed on the I/O Channel slot of your computer, you must check the port

addresses which are used by them.  And if the address, which is set by the

/B:xxx in CONFIG.SYS file and by the switch settings of the host adapter for

CD-ROM drive, is used by another peripheral, you must change it to different

address.

Notice again that the base address of CONFIG.SYS file and switch settings of

the host adapter must be same.  Refer to CUSTOMIZATION of README.DOC file for

changing CONFIG.SYS file.

#

page 12

2-2. IRQ CHANNEL.

To avoid the conflict between Sony MS-DOS CD-ROM Extensions and other

peripherals, this driver does not use any IRQ channe, even if you specify

an IRQ channel.

###page 13

The following table is just for reference of the IRQ channel.

IRQ #    IBM PC/AT               IBM PC/XT

--------------------------------------------------------------------

0     TIMER                   TIMER

1     KEY BOARD               KEY BOARD

2     CONTROLLER #2         * RESERVED

3   * SERIAL PORT #2        * SERIAL PORT #2

4     SERIAL PORT #1          SERIAL PORT #1

5   * PARALLEL PORT #2        HARD DISK DRIVE

6   X FLOPPY DISK DRIVE     X FLOPPY DISK DRIVE

7     PARALLEL PORT #1        PARALLEL PORT #1

--------------------------------------------------------------------

"*" may be used, "X" can not be used.

#

page 14

2-3. DMA CHANNEL.

To avoid the conflict between Sony MS-DOS CD-ROM Extensions and other

peripherals, it is better not to use any DMA channels.  When you have installed

Sony MS-DOS CD-ROM Extensions using INSTALL.BAT, the default setting of DMA

channel is "not used."

If you intend to use DMA, you must check which channel(s) is (are)

available.   To set the DMA channel, you have to specify the DMA channel number

by /T:drq in CONFIG.SYS file.  And you must also set DMA transfer mode by /M:D

in CONFIG.SYS file.

Notice that you must set the DIP switches on the host adapter to the number

you set in CONFIG.SYS file.   Refer to CUSTOMIZATION of README.DOC file.

The following table is just for reference.

DMA #    IBM PC/AT               IBM PC/XT

--------------------------------------------------------------------

0     RESERVED                D-RAM REFRESH

1   * SDLC                  * SDLC

2   X FLOPPY DISK DRIVE     X FLOPPY DISK DRIVE

3   * RESERVED                HARD DISK DRIVE

--------------------------------------------------------------------

"*" may be used, "X" can not be used.

SDLC stands for Synchronous Data Link Control.

###page 15

2-4. EXECUTABLE PATH AND SUB-DIRECTORY OF THE DEVICE DRIVER.

If you have installed Sony MS-DOS CD-ROM Extensions on a sub-directory with-

out using ADINST.BAT, you must append the followings to AUTOEXEC.BAT and

CONFIG.SYS files.   Otherwise the error messages appears when boot.

1. The executable path name for MSCDEX.EXE in AUTOEXEC.BAT file.

****** An example of AUTOEXEC.BAT file ********

PATH =  C:\;C:\Sub_Dir

MSCDEX /D:MSCD220

2. The sub-directory, on which the device driver exists, in CONFIG.SYS file.

****** An example of CONFIG.SYS file ********

DEVICE=C:\Sub_Dir\SONY_CDU.SYS /D:MSCD220

2-6. DEVICE NAME.

The device name both in AUTOEXEC.BAT and CONFIG.SYS files must be same.

The device name is specified by "/D:device_name" in the two files.

#

page 16

CHAPTER 3. ENVIRONMENT VARIABLE AND DISK SPACE.

When the MS-DOC CD-ROM Extensions does not run well, even if everything is

correct, check the environment variable.  If the environment variable is

insufficient, the following troubles may occur.

1. The message of "Out of environment space" is displayed on the screen.

2. Although an executablle path name is appended in AUTOEXEC.BAT file, and

MSCDEX.EXE is copied in the sub-directory you created, the MSCDEX never

runs.  In this case, a message of "Bad command or file name" is displayed

on the screen.

To fix the problems, you must chahge the environment variable in CONFIG.SYS

file.

***  An example of setting the environment variable in CONFIG.SYS file. ****

shell = c:\command.com c:\ /p /E:1024

Refer to the manual of your operating system for detail.

If available room on the target disk is less than 62 K bytes, the message of

"Insufficient disk space" appears on the screen during installation.   In this

case you can not install Sony MS-DOC CD-ROM Extensions.  Make a sufficient room

on the target disk.

************************ END OF CHCKLIST.DOC FILE ******************************

#