This README.DOC gives you more detailed informations about the installation of

Sony MS-DOS CD-ROM Extensions, compared with Installation Manual.

Make a hard copy of this file using PRINT command, if you need it.

For easy installation onto the hard disk drive C, use one of below.

*INSTALL1: For one CD-ROM Drive

*INSTALL2: For two CD-ROM Drives

*INSTALL3: For three CD-ROM Drives

*INSTALL4: For four CD-ROM Drives

Usage:  A:\>INSTALL# AT(or XT) [Sub_Dir]	(# = 1, 2, 3 or 4)

Your computer type (AT or XT) must be specified.   If Sub_Dir is ommitted,

Driver and Software will be installed onto the root directory.

When you wish to know how to set switches on your host adaptor card, type

CDB_SW and Enter.  On screen image of Sony CD-ROM host adaptor card will

appear.

###page  1

************************ TABLE OF CONTENTS ****************************

CHAPTER NUMBER AND THE TITLE                                              PAGE

CHAPTER 1. INTRODUCTION   ................................................. 2

CHAPTER 2. PREPARATION   .................................................  3

2.1. PREPARATION OF THE TARGET FLOPPY DISK  ...........................  3

2.2  CONFIGURATION OF YOUR COMPUTER   .................................  3

2.3. INSERTION OF THE LIBRARY DISK AND THE TARGET DISK   ..............  4

CHAPTER 3. BASIC INSTALLATION   ..........................................  5

CHAPTER 4. ADVANCED INSTALLATION   .......................................  6

4-1. ON A SUB DIRECTORY   .............................................  7

4.2. ON A FLOPPY DISK BOOTED FROM THE HARD DISK C   ...................  8

CHAPTER 5. CUSTOMIZATION   ...............................................  9

5.1. CHANGING THE PARAMETERS IN AUTOEXEC.BAT FILE   ................... 10

5.2. CHANGING THE PARAMETERS IN CONFIG.SYS FILE   ..................... 12

CHAPTER 6. STRUCTURE OF THE LIBRARY DISK   ............................... 16

6-1. DIRECTORIES AND FILES   .......................................... 16

6-2. STREAM OF BATCH FILES   .......................................... 17

END OF README.DOC FILE  .................................................. 17

#

page  2

CHAPTER 1. INTRODUCTION.

Welcome to the wonderful world of Sony MS-DOS CD-ROM Extensions.

Prior to installation, make a backup copy of this Library disk using

DISKCOPY command.  Store the master disk in a safe place and use the copy for

installation.

Even if the supplied Library disk is write-protected, you should not protect

the backup copy of the Library disk.   Otherwise you can not install Sony

MS-DOS CD-ROM Extensions.

This installation utility consists of batch files, screen images

and execution programs.   The structure of this Library disk and the stream

of the batch files are described in CHAPTER 6 of this document.

Type "INSTALL" and press Enter key.  You can install Sony MS-DOS CD-ROM

Extensions on a target disk by following on-screen instructions.

The installation manual describes about basic installation, however you can

install Sony MS-DOS CD-ROM Extensions using another method.  Refer to "ADVANCED

INSTALLATION" of this document.

###page  3

CHAPTER 2. PREPARATION.

2.1. PREPARATION of THE TARGET FLOPPY DISK.

If you wish to install the MS-DOS CD-ROM Extensions on your floppy disk, it

is necessary to prepare a bootable floppy disk.  The disk should be formatted

as instructed below, and DOS is transferred to the disk.   We call the disk

"Target floppy disk" in the document.

C:\>FORMAT A: /S  and <Enter>

2.2. CONFIRMATION OF YOUR COMPUTER.

Before installation, confirm the followings.

Type of your computer.  IBM PC/AT or PC/XT?

The number of floppy disk drives installed on your computer.

With hard disk drive or not.

The drive letter which the MS-DOS was booted from (A or C).

#

page  4

2.3. INSERTION OF THE LIBRARY DISK AND THE TARGET DISK

After booting, you have to insert the Library disk and the target disk as

follows.

CASE.1:  If your computer has one floppy disk drive and no hard disk

drive, insert the Library disk into drive A.

CASE.2:  If your computer has two floppy disk drives and no hard disk

drive, insert the Library disk into  drive B.

(The boot disk has been inserted in drive A.)

CASE.3:  If your computer has a hard disk drive and one floppy disk

drive, insert the Library disk into drive A.

CASE.4:  If your computer has a hard disk drive and two floppy disk

drives, insert the Library disk into drive B and

insert the target floppy disk into drive A.

(If you  install the MS-DOS CD-ROM Extensions on the hard disk

drive, you do not have to insert the target floppy disk into

drive A, and you can insert the Library disk into drive A or B.)

###page  5

CHAPTER 3. BASIC INSTALLATION.

Change the logged drive from the boot drive to the drive with Library disk.

Then type INSTALL and the Enter key at the DOS prompt.  The installation

utility will start and press any key until the following messages appear on

the screen.  Continue responding to the message.

Type one of the followings and press the Enter key at the

DOS prompt.

The '??' is your computer type, PC/AT or PC/XT.

Type like 'CH1 AT Sub_Dir'.

CH1 ?? {Sub_Dir}  ; Install on a hard disk.

AF2 ?? {Sub_Dir}  ; Install on a floppy disk using double floppy disk

drive system.

AF1 ?? {Sub_Dir}  ; Install on a floppy disk using single floppy disk

drive system.

#

page  6

CHAPTER 4. ADVANCED INSTALLATION.

Type "ADINST" and press Enter key, then the following messages appear.

*******************  ADVANCED INSTALLATION. **************************

Type one of the followings and press Enter key. If you omit the SUB_DIR,

Sony MS-DOS CD-ROM Extensions will be installed on the root directory.

CH1 AT(or XT) {Sub_Dir}  ;Booted from the hard disk C.

Install on SUB_DIR of the hard disk.

CF1 AT(or XT) {Sub_Dir}  ;Booted from the hard disk C.

Install on SUB_DIR of floppy disk using

single floppy drive system.

CF2 AT(or XT) {Sub_Dir}  ;Booted from the hard disk C.

Install on the SUB_DIR of floppy disk using

double floppy drive system.

AF1 AT(or XT) {Sub_Dir}  ;Booted from floppy disk A.

Install on the SUB_DIR of a floppy disk

using single floppy disk drive system.

AF2 AT(or XT) {Sub_Dir}  ;Booted from floppy disk A.

Install on the SUB_DIR of a floppy disk

using double floppy disk drive system.

###page  7

4-1. ON A SUB DIRECTORY.

If you wish to specify the sub-directory, type like "A:\>CH1 AT SUB_DIR"

and Enter key.  Then, the batch files and SHELL.COM do the followings;

1. Make a sub-directory of  SUB_DIR on the target disk.

2. Make backup files of AUTOEXEC.BAT and CONFIG.SYS with extension name of

"BAK" on the sub-directory, if the target disk is the hard disk C.

3. Append or make the executable path name of the "DRIVE_LETTER:\SUB_DIR;" in

AUTOEXEC.BAT file.

4. Append the sub-directory, on which the device driver exists, in CONFIG.SYS

file.

*** An example of AUTOEXEC.BAT file after the installation. ***

path=C:\Sub_Dir;%path%

MSCDEX /D:MSCD220 /M:4 /L:S /V

*** An example of CONFIG.SYS file after the installation. ***

DEVICE = C:\Sub_Dir\SONY_CDU.SYS /D:MSCD220 /U:1 /B:340 /M:H /Q:* /T:*

LASTDRIVE=Z

#

page  8

4.2. ON A FLOPPY DISK BOOTED FROM THE HARD DISK C.

Someone may wish to install the MS-DOS CD-ROM Extensions on a floppy disk

when DOS was booted from the hard disk C.  An advantage of this is that you can

install the MS-DOS CD-ROM Extensions without changing the hard disk environment,

and that you can use the floppy disk when you need to run the MS-DOS CD-ROM

Extensions.

Follow the sequence below for installation.

1. Boot your computer from the hard disk drive C.

2. If your computer has two floppy disk drives, insert the Library disk into

drive B, and the target disk into drive A.

If your computer has one floppy disk drive, insert the Library disk into

drive A.

3. Change the logged drive to the drive with Library disk.

4. Type below and Enter key, then the installation utility will start.

B:\>CF2 AT(or XT) {Sub_Dir}    ; Double floppy disk drive system.

A:\>CF1 AT(or XT) {Sub_Dir}    ; Single floppy disk drive system.

NOTE: In case of only one floppy disk drive system, you have to repeat to

insert the library disk and the target disk by following on-screen messages.

###page  9

CHAPTER 5. CUSTOMIZATION.

Sony MS-DOS CD-ROM Extensions has some parameters in AUTOEXEC.BAT and

CONFIG.SYS files.

After installation the MS-DOS CD-ROM Extensions, you may customize them by

using your favorite editor.

Check the parameters first by using TYPE command as follow.  Each parameter

is set after "/", such as /L:S.

A:\>TYPE AUTOEXEC.BAT    ; You can find the string below on the screen.

MSCDEX /D:MSCD220 /M:4 /L:S /V

A:\>TYPE CONFIG.SYS      ; You can find the two strings below on the screen.

DEVICE=SONY_CDU.SYS /D:MSCD220 /U:1 /B:340 /M:H /Q:* /T:*

LASTDRIVE=Z

#

page 10

5.1. CHANGING THE PARAMETERS IN AUTOEXEC.BAT FILE.

To start the MS-DOS CD-ROM Extensions when you boot the computer, you must

include the following string in AUTOEXEC.BAT file.  You can omit parameters

in two square braces.

MSCDEX /D:device_name [/M:number /L:drive_letter /V]

Change each parameter according to the followings if necessary.

NOTE: Reboot by pressing Ctrl-Alt-Del keys after you changed the parameters.

/D:device_name

You can change the device_name using a maximum of eight letters, such as

/D:CDROM001.  You must write the same device_name in the CONFIG.SYS file too,

otherwise the MS-DOS CD-ROM Extensions never works.

The default setting is MSCD220.

###page 11

/M:number

This parameter specifies the number of sector buffers to be used for

caching the path table of a CD-ROM disc.

Each drive must have a minimum of 4 buffers, but the larger number is,

the better the performance will be.

If you delete this parameter, the default value of 4 is set.

/L:drive_letter

You can write C to Z that is not in use by other peripherals, however make

sure which drive letters are already used by other peripherals.   You should

not set the drive letter used by other peripherals as the drive_letter.

If you omit this parameter, the letter which comes after the one assigned

to the current last drive.    The default setting is S.

/V

If this parameter is written, you can see the information about memory

usage when you boot the computer.  You don't have to set the parameter if

not necessary.

#

page 12

5.2. CHANGING THE PARAMETERS IN CONFIG.SYS FILE.

To start the MS-DOS CD-ROM Extensions, you must include the following

strings in CONFIG.SYS file.  You can omit parameters and a string in two square

braces.

DEVICE=SONY_CDU.SYS /D:device_name [/U:un /B:xxx /M:mode /Q:irq /T:drq]

[LASTDRIVE=last_letter]

Change each parameter according to the following  explanations.

NOTE:	This device driver ignores IRQ channel.

Reboot by pressing Ctrl-Alt-Del keys after you changed the parameters.

###page 13

LASTDRIVE=last_letter

This string sets the maximum number of drives that you may access.  The

last letter represents the last valid drive letter that MS-DOS may accept.

Set the last_letter regarding the drive_letter specified by "/L:drive_letter"

in AUTOEXEC.BAT file and the unit number specified by "/U:un" in CONFIG.SYS

file as following example.   The default setting is W.

the drive_letter     the un      the last_letter

-------------------------------------------------------

S                    1           T

S                    2           U

S                    3           V

S                    4           W

E                    1           F

-------------------------------------------------------

If your computer is on a network, set the last_letter which is larger than

the total number of peripherals in your system, so that new net work

peripherals can be added after booting.

Notice though this string is allowed to be omitted, it is better to set

the last_letter to avoid allocation error of the drive letter for CD-ROM drives.

#

page 14

/D:device_name    See the CHAPTER 5.1.

/U:un

The 'un' specifies the unit number of CD-ROM drives connected in a daisy

chain.  You can write 1 to 4 as the un, according to the number of CD-ROM

drive(s) connected in a chain.

If you omit this parameter, the default value of 1 is set.

/B:xxx

The 'xxx' specifies the base address of the CD-ROM host adapter with a

hexadecimal number.   Make sure that this number and the switch settings on

the host adapter are same.  Also make sure that the address is not used by

other peripherals.

If you omit this, the default value of 340h is set.

/M:mode

The 'mode' specifies the data transfer mode.  D for DMA transfer P for

software transfer and H for high speed software transfer (for fast CPU.)

Notice if you specify "D", you have to specify the number of DRQ channel

by /T:drq parameter.  Refer to the next page.

If you omit this, the default value of H(PC/AT) or P(PC/XT) is set.

###page 15

/Q:irq

The driver does not use the IRQ, even if specified by this switch.

/T:drq

The 'drq' specifies the DRQ channel number for DMA transfer.  When you

select the DMA transfer using /M:D above, you must select a DRQ channel number.

You can select 1, 2 or 3 as the 'drq'.   Make sure that this number and

the switch settings on the host adapter are same.   Also make sure that the

channel is not used by other peripherals.

If you do not use the DMA transfer, omit this or write '/T:*'.

#

page 16

CHAPTER 6. STRUCTURE OF THE LIBRARY DISK.

6-1. DIRECTORIES AND FILES.

ROOT-----   BATCH FILES    (For Installation of MS-DOS CD-ROM Extensions.)

|        README.DOC     (Text File of this document.)

|        README.EXE     (Display this document on the screen.)

|        CHCKLIST.DOC   (Text file of the check list for trouble shoot.)

|        CHCKLIST.EXE   (Display the CHCKLIST.DOC file on the screen.)

|

|

+--------BIN----- MSCDEX.EXE     (MS-DOS CD-ROM Extensions program.)

SONY_CDU.SYS   (Sony MSCDEX Device driver.)

CDPLAY.EXE     (An example utility for CD Audio disc.)

MSCDINFO.EXE   (Display status of the installation.)

CDB_SW.COM     (On screen instruction of switch

settings.)

LOCK/UNLOCK/EJECT.COM

###page 17

6-2. STREAM OF BATCH FILES.

INSTALL.BAT --> SEL_SEQ.BAT -->   CH1.BAT || --> LAST_MSG.BAT

AF1.BAT ||

AF2.BAT ||

ADINST.BAT  -->   CH1.BAT ||  --> LAST_MSG.BAT

CF1.BAT ||

CF2.BAT ||

AF1.BAT ||

AF2.BAT ||

CDB200.BAT, CDB220.BAT, CDB230.BAT and CDB231.BAT are independent files.

Make a hard copy of this file using PRINT command, if you wish.

************************ END OF README.DOC FILE ******************************

#